BOOKEND

# Project

Procedures:

- Obtain Material  1 1” x  4” x  30” Piece of # 2 Pine

- Measure and layout materials as depicted on drawing.

  - Layout the Base 9 ¾”

  - Layout Upright 6”

- Attach Base to Upright using 6d finish nails and wood glue.

  - Be sure that the base is 90º to the upright

- Layout the angles for the gusset by laying your remaining piece of material across the Base and Upright so that the top edge of the gusset is lined up with the corners of the Base and Upright.

- Mark the Gusset so at the ends of the base and upright.

- Use the back saw to cut the gusset at the correct angles.

- After you have cut your gusset mark on your base and upright where the notches need to be made to receive your gusset.

- Attach the gusset to the base with woodscrews.

  - Be sure to use the countersink bit for your screw holes.

  - Layout your initials on the gusset and remove wood with router.

- Fill your screw and nail holes with wood filler

- Sand and finish your project.