Note Pad Holder

Name: __________________________

Date: __________________________

## Description:

The note pad holder is a cold metal project that involves layout skills and sheet metal fabrication skills

## Materials:

3–1/8“x ¼” Aluminum Pop Rivet

18 ga. Galvanized Sheet Metal

1–1/8”x1” Flat Hot Rolled Mild Steel

1–1/8“x1”x1” HR Angle Iron

2–1”x ¼”-20 (NC) Pan Head Screw

2– ¼” Split Lock Washer

2– ¼”-20 (NC) Wing Nut

## Tools:

Steel Tape Measure

Combination Square

Scratch Awl/Sharpie

Sheet Metal Shear

Mill Files (Bastard & Smooth)

Abrasive Cut-off Saw

Center Punch

Ball Peen Hammer

Pop Rivet Tool

Drill Press

#30 Drill Bit

¼” Twist Drill

Locking Pliers

## Procedure:

- Layout the 5½”x7” main base on 26ga galvanized sheet metal.  Cut using a sheet metal shear.

- 2.	Measure a 5½“ long  piece of 1/8”x1” flat steel for the top plate strip.  Measure a 5½“ long piece of 1/8”x1”x1” angle iron for the bottom support.  Cut both pieces using an abrasive cut off saw.

- File rough or uneven edges smooth or even with mill file.

- Layout two locations for drilling outside screw-holes on the top plate strip front using a center punch.  Line-up and securely clamp together all three pieces of material with locking pliers.

- Drill two outside holes using a ¼” drill and drill press.  Remove any burs with a file.

- Layout three locations for drilling inside pop rivet-holes on the main base plate front using a center punch.  Line-up and securely fasten together the main base and support for drilling.

- Drill three inside holes using a #30 drill bit and drill press.  Remove any burs with a file.

- Attach the main base plate and supporting angle iron using three 1/8”x ¼” pop rivets and a pop rivet tool.  Drive down the metal tight against rivet head with a ball peen hammer, if necessary.

- Assemble note pad holder inserting a 1”x ¼”-20NC pan head screw through each outside hole along with a ¼” split lock washer.  Thread on a ¼”-20NC wing nut to each.

## Cutting List:

| Description | Quantity | Size | Material |
| --- | --- | --- | --- |
| Main Base Plate | 1 | 5½“x7” | 18 ga. Galvanized Sheet Metal |
| Top Plate Strip | 1 | 5½“ | 1/8”x1” Flat Cold Rolled Mild Steel |
| Bottom Support | 1 | 5½“ | 1/8”x1”x1” Angle Iron |

## Notes:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Photo/Drawing:

## Note Pad Worksheet

Name: ________________________

Date: ________________________

- The main base plate of the notepad holder is ________ by ________.  (1 pt)

- Calculate the distance of each of the two outside 5/16” drilled holes from the project edges, and note on given plan.  The center of each hole is measured at ½” from the top of the note pad holder, and ________ from the side, both right and left, project edges.  (1 pt)

- Name the three larger machine tools used to complete this project.  (3 pts)

- ______________________            ______________________            ______________________

- The tool used to smooth rough edges of metal is called a ______________________.  (1 pt)

- List the following steps in order of the recommended method used to layout and drill a hole with a drill press.  (2 points)

- ___ At a 90°-angle to material surface, drill hole completely through applying light, constant pressure on hand feed lever

- ___ Place and firmly secure materials for drilling to the drill platform

- ___ According to plan, measure and mark locations of holes to drill with a center punch

- ___ Select desired drill bit to put in drill press chuck and a piece of wood for protective backing

- 5.	The difference in the diameter of the 1/8” pop rivets (________) and the #30 drill bit (________) is __________.  This difference helps to eliminate a(n) ______________________.  (2 pts)

## Grading Rubric:

| CRITERIA | POSSIBLE | SCORE |
| --- | --- | --- |
| Worksheet Questions | 10 |  |
| Assembly (properly assembled, ordered lay of pieces) | 5 |  |
| Measurement – height & width | 5 |  |
| Rivets (placement, cleanliness, tight) | 5 |  |
| Screws (placement, cleanliness) | 5 |  |
| Workmanship (straight cuts, not sharp edges, flat/level) | 5 |  |
| TOTAL POINTS : | 35 |  |

# Note Pad Holder -- Teachers Notes:

## Agricultural Standards Met:

B1.0	Students understand personal and group safety:

B1.1	Practice the rules for personal and group safety while working in an agricultural mechanics environment.

B1.2	Know the relationship between accepted shop management procedures and a safe working environment.

B5.0	Students understand agricultural cold metal processes:

B5.1	Know how to identify common metals, sizes, and shapes.

B5.2	Know basic tool-fitting skills.

B5.3	Know layout skills.

B5.4	Know basic cold metal processes (e.g., shearing, cutting, drilling, threading, bending).

B5.5 	Complete a cold metal project, including interpreting a plan, developing a bill of materials, selecting materials, shaping, fastening, and finishing.

## Objectives:

By properly completing this project, students will be able to:

- Read a plan and to layout dimensions.

- Safely and properly use a power machine tool, such as a drill press, sheet metal shear and abrasive cut off saw.

- Perform basic sheet metal fabrication.

## Alternative Tools/Methods/Materials:

- Make larger in size (for 8½”x11” paper)

- Use various tools for cutting: hydraulic shear, chop saw, hacksaw, iron worker, Beverly shear, tin snips, etc.

- Use lighter/heavier, thinner/thicker angle iron or flat steel

- Use different kinds or sizes of bolts, screws or nuts (bigger drill bits = less likely to be broken)

- Tap holes for screws

- Replace flat cold rolled mild steel top holding strip with sheet metal

- Replace angle iron and top flat strip with wood and/or use wood for base material

- Fold edges of sheet metal

- Add metal strip to bottom of front face (design)

- Replace angle iron with a hanger or hook

- Make entire project out of aluminum

- Drill more holes (makes project adjustable for various sizes of paper)

- Replace pop rivets with spot welding or with countersunk screws

## Safety Review:

- Abrasive cut off saw/cutting metal

- Sheet metal shear/shearing metal

- Drill press (floor)/drilling holes in metal

## Project Time:

| Demonstration: | 40 minutes |
| --- | --- |
| Build: | 3 hours |

## Demonstration Notes

- Measure twice before you cut.

- Remember to file or smooth rough and sharp edges.

- Layout/measure material at machines and/or drilling to minimize back and forth traveling.  Demo the use of the combination square for layout.

- Emphasize correct drill speed for drilling metal.

- Clamp material pieces together locking pliers facing upwards, minimizing interference in accurately securing and leveling project on drill press platform before drilling.  By drilling the clamped pieces you insure the holes will align.

- Remember: metal will be hot after cutting with an abrasive cut off saw and after drilling.

- Using a #30 drill bit (.1285” diameter) to make holes for 1/8” pop rivets (.125” diameter) eliminates an interference fit.  Since the hole is slightly larger (difference of .0035”), inserting the pop rivet is more efficient, requires less time to try and push it in, and allows for more productivity.

- Push pop rivet and tool firmly against material before squeezing to ensure rivets are flush against metal.

- Unclamp pieces after drilling outside ¼” holes, and attach main base plate to supporting angle iron with screws for drilling inside pop rivet holes.

- Pound down the rivet heads to make them as flush as possible against the metal.

- Lock washer is placed near end of screw next to wing nut.

- Workmanship should be emphasized.

## Bill of Materials:

Project from: Mr. Patterson, Ripon High School

Plan by: Lindsey Pahl