Electrical -Non-Metallic Cable Wiring

Name: ________________________

Date: ________________________

## Description

This activity will provide students with the knowledge and skill of wiring a three way switch with non-metallic cable to the National Electric Code.

## Materials:

14/2 Nonmetallic sheathed cable

14/3 Nonmetallic sheathed cable

Wire Nuts

Plastic Boxes (3)

Lamp (1)

3 way switch (2)

Wood Base

NM Staples

## Tools

NM Cable Cutter

Wire Strippers

Long Nose Pliers

Screwdriver

Claw hammer

## Directions:

- Review the rules for wiring (text book).  Key points are:

  - Color coding of wire and terminals.  Green=ground, white=neutral, other colors are hot. Brass terminals are hot wires, silver terminals are neutral, green terminals are ground.

  - 6” of free conductor.

  - Use of wire nuts:  tight and no bare wire showing.

  - Grounding:  boxes and devices.

  - Proper connection to a screw terminal:  Clockwise, loop ¾ around, no insulation under the screw, no bare wire past the device.

- Identify all the parts of the project by name.

- Neatly diagram your circuit on the worksheet. Show wire connections with a line and label each line with proper wire color.

- Pull cable of the appropriate type to each box (remember 6” free conductor).

- Fasten the cable with a staple.

- Strip the cable sheath.

- Strip the wires.

- Install the devices as demonstrated (follow your circuit diagram).

- Observe color-coding, grounding, and all other National Electrical Code (NEC) rules.

- Secure the devices to the boxes with screws for testing.

- Attach a temporary power cord to your source wires with wire nuts. Observe color-coding.

- Using the testing circuit breaker box, plug in your board and test the circuit using a lamp and duplex tester. (Circuit grading is done here)

- Disconnect board.

- Remove the screws securing the devises to the boxes so the project can be graded.

## Write your name on board and turn in for final grading with the worksheet.

## Drawing/Photo:

14/2 NM Cable

Lamp (A)

3 way Switch (B)

3 way switch (C)

Plastic Box

14/3 NM Cable

14/2 NM Cable

Notes:

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

## Grading Rubric:

| Criteria | Possible | Score |
| --- | --- | --- |
| Correct circuit (live test) | 8 |  |
| Correct Grounding | 2 |  |
| Correct usage of wire colors and terminal color coding | 2 |  |
| 6" of free conductor | 2 |  |
| Screw terminals neat and correct (wire clockwise, etc). | 2 |  |
| Correct use of wire nuts (no bare conductor, not loose). | 2 |  |
| Workmanship (neatness) | 2 |  |
| TOTAL | 20 |  |

____________________________________________________________________________________

# Electrical Work Sheet

Name: ________________________

Date: ________________________

- What color is the “neutral” wire? __________________________

- What colors are the “hot” wires? __________________________

- What color is the “ground” wire? __________________________

- What are the “traveler” wires? ____________________________________

- How much “free conductor” is required to extend from the box? ____________

- The black dots represent the screw terminals on the devises.  Label each dot with the color of the screw (e.g., brass, silver).  Connect the black dots on the diagram below to illustrate how you will wire your project.  Label each wire color.

Lamp

3 way

Switch

White

Black

bare

3 way

Switch

## Electrical Teaching Notes:

## Agricultural Standards Met:

- 4.0	Technology. Students know how to use contemporary and emerging technological resources in diverse and changing personal, community, and workplace environments:

- 4.6	Differentiate among, select, and apply appropriate tools and technology.

- 5.0	Problem Solving and Critical Thinking.  Students understand how to create alternative solutions by using critical and creative thinking skills, such as logical reasoning, analytical thinking, and problem-solving techniques:

- 5.1	Apply appropriate problem-solving strategies and critical thinking skills to work-related issues and tasks.

- 5.3	Use critical thinking skills to make informed decisions and solve problems.

- 6.0	Health and Safety. Students understand health and safety policies, procedures, regulations, and practices, including the use of equipment and handling of hazardous materials:

- 6.1	Know policies, procedures, and regulations regarding health and safety in the workplace, including employers’ and employees’ responsibilities.

- 6.2	Understand critical elements of health and safety practices related to storing, cleaning, and maintaining tools, equipment, and supplies.

- 6.4	Maintain safe and healthful working conditions.

- 6.5	Use tools and machines safely and appropriately.

- 6.6	Know how to both prevent and respond to accidents in the agricultural industry.

- B1.0	Students understand personal and group safety:

- B1.1	Practice the rules for personal and group safety while working in an agricultural mechanics environment.

- B1.2	Know the relationship between accepted shop management procedures and a safe working environment.

- B3.0 Students understand the basic electricity principles and wiring practices commonly used in agriculture

- B3.1 	Understand the relationship between voltage, amperage, resistance, and power in single-phase alternating current (AC) circuit.

- B3.2	Know how to use proper electrical test equipment for AC and direct current (DC)

- B3.3	Know Analyze and correct basic circuit problems (e.g. open circuit, short circuit, incorrect grounding)

- B3. 4	Understand proper basic electrical circuit and wiring techniques with nonmetallic cable and conduit as defined by the National Electric Code

- B3.5	Interpret basic agriculture electrical plans.

## Alternate Tools/Materials:

This project could have been laid out on variety of boards. Simpler or more complex circuits can be made.

## Safety Review:

High voltage circuits will be tested.

## Project Time:

| Demonstration: | 15-20 minutes |
| --- | --- |
| Build: | 2 hours |

## Demonstration Notes:

(Tips, methods to stage the project)

- Review the following rules:

  - Where NM cable can be used (residential)

  - 6” free conductor

  - Color coding of wires and screw terminals.  Discuss marking of white wires as “hot” with tape when needed in NM wiring.

  - Proper length to strip wire for wire nuts and screw terminals

  - Wiring of screw terminals (clockwise)

  - Grounding  with NM cable

  - Stapling of NM cable

- Review the wiring of three way switch.  How to identify the “travelers” that connect the two switches.

- Pull cable to the boxes

- Staple (within 6” of box)

- Demonstrate stripping of the cable sheath

- Attach ground wires to each other.

- Tip:  Make screw eyes by bending backwards first.

- Install devices

- Tip:  Show how to “fold” wire into the box.

- Test circuit

## Bill of Materials

(Excel, update with local prices)